Cube User Programs for winxx

Jens Bribach, GIPP – Jan. 2014

- User Package

Cube_SetUp.exe

CubeMonitor.exe

CubePlotter.exe

CubeCopy.exe

config.300

config.txt

- Program Overview

  - Cube SetUp

Cube_SetUp.exe

- Selects and/or generates parameter set, e.g. default.hea

- Calculates battery life time for different types

- Calculates Cube Memory life time

- Finally generates  config.txt (to be copied later to the Cube Memory Root Dir)

File config.300 (read-only) contains the Default, opened when pressed ‘New SetUp’.

Additionally there are SetUp Files for typical recording conditions

- broadband100.hea (gain = 1; sampling rate = 100 sps)

- mark100.hea (gain = 16; sampling rate = 100 sps)

- mark200.hea (gain = 16; sampling rate = 200 sps)

- geo100.hea (gain = 64; sampling rate = 100 sps)

- geo200.hea (gain = 64; sampling rate = 200 sps)

  - Cube Monitor

CubeMonitor.exe

- Displays Cube Boot Process (when connected before PowerOn)

- Displays Cube Time Series

- Shows Peak-to-Peak Amplitude Values

- Selectable HighPass (recommended for viewing Broad Band Seismometers)

CubePlotter.exe

- CubeMonitor, plotting multiple lines per channel

The directory has to contain the currently used config.txt (when connected to a just running Cube).

  - Cube Copy

CubeCopy.exe

- Checks for available Cube Memory as well as additional USB drives

- Copies the daily generated data directories to one or two external (USB) drives

- Formats – on request – the Cube Memory

- Uploads – on request – a new configuration file config.txt

- Program Handling

  - Cube SetUp

- Start program

- Select available project file by double click or press ‘New SetUp’

- When pressed ‘New SetUp’

.	Enter new project name and apply it by double click, e.g. TEST100

- Apply changes

- Press ‘Save SetUp’, the setup will be saved as e.g.  TEST100.hea

- Press ‘Save config.txt’, the new configuration file config.txt will be saved.

  - Cube Monitor (unsuitable for external Cube power supply)

- Connect TERM cable to Cube and PC/Laptop

- Start program

- Select serial port ( hardware manager, ‘COM and LPT’: look for ‘USB Serial Port’)

- Select ‘HighPass’ or ‘RST Offset’ if necessary

- Press ‘START’

  - Cube Copy

- Connect USB cable to Cube and PC/Laptop

- Start program, the program checks for available Cube and USB drives

- Change destination drives if necessary

- Build new directory if necessary

- Check checkboxes (help appears at mouse position)

- Press ‘Copy All’ (all data copied)

or ‘Copy Last’ (last day of data copied)

or ‘Erase’ (Cube memory formatted without copying)

- Connect to next Cube

- Press ‘Refresh’ to see the new directories

- Continue at item 3

  - Configuration Upload

The Cube Configuration File config.txt is situated in the root directory of the Cube memory sD-card. So one can upload it after connecting USB cable to Cube and PC/Laptop.

The relevant drive name is CUBE1 or CUBE3, followed by the Cube serial number, e.g. CUBE3-612.

It is advisable to build new configuration files by use of the above described program CubeMonitor.exe.